Task 1.1P: Switching ON Lights

Modular Programming Approach

SIT210 — Embedded Systems Development

# 1. GitHub Repository

The complete code for this task is available in the GitHub repository at the following link:

Repository Link: https://github.com/[your-username]/SIT210/tree/main/Task1.1P

The repository folder Task1.1P contains:

- Task1.1Lights_ON.ino — the main Arduino source code file

- README.md — system description and code documentation

# 2. Circuit Schematic

The circuit schematic was created using Tinkercad and shows the wiring of two LEDs and a push button connected to the Arduino Nano 33 IoT.

Tinkercad Schematic Link: https://www.tinkercad.com/[your-tinkercad-link]

## Wiring Summary

| Component | Arduino Pin | Notes |
| --- | --- | --- |
| Porch LED (+) | D5 | Via 220Ω resistor to GND |
| Hallway LED (+) | D3 | Via 220Ω resistor to GND |
| Push Button | D2 | Other leg to GND |
| LED (-) legs | GND | Through 220Ω resistors |

# 3. Demonstration Video

A short video demonstrating the working system — including the Arduino board, breadboard circuit, and both LEDs responding to the push button — is available at the following link:

Video Link: https://www.youtube.com/[your-video-link]

The video shows the push button being pressed, the porch LED activating for 30 seconds, followed by the hallway LED activating for 60 seconds, and both turning off automatically.

# 4. Modular Programming Discussion

In this task, modular programming was applied by breaking the light control system into four dedicated functions, each responsible for a single, clearly defined task.

The function setupPins() handles all pin configuration in one place. If the circuit wiring ever changes — for example, moving the porch LED to a different pin — only this one function needs to be updated, rather than searching through the entire program.

The function isButtonPressed() isolates the input-reading logic. This is useful because if the hardware changes from a push button to a slider switch or even a sensor, the rest of the program does not need to change — only this function is modified.

The functions activatePorchLight() and activateHallwayLight() each independently manage one light. This separation is particularly valuable in the context of Linda's system: the porch light has a 30-second duration and the hallway light has a 60-second duration. By keeping these as separate modules, adjusting one light's timing does not risk accidentally breaking the other.

The loop() function itself remains clean and readable — it simply asks 'is the button pressed?' and if so, calls the relevant light modules in sequence. This makes the overall program logic easy to follow and maintain, which is critical when expanding the system to include more rooms or features in the future.

# 5. Design Problems Discussion

While the solution works for its intended purpose, there are several design limitations worth noting:

- Blocking delay() calls: The use of delay(30000) and delay(60000) completely freezes the Arduino for the duration of each delay. During this time, no other inputs can be read — meaning if Linda presses the button again, or if another sensor triggers, the system cannot respond until the delay finishes.

- Sequential execution: The porch light must fully complete its 30-second cycle before the hallway light begins. In a real scenario, both lights would ideally turn on simultaneously rather than one after the other.

- No user feedback: There is no visual or audio confirmation that the system registered Linda's button press. If the button is pressed while a delay is running, the press is silently ignored, which could be confusing for an elderly user.

- Button debouncing: A basic 50ms debounce delay is used, but this is not robust. Mechanical buttons can produce multiple rapid signals from a single press (bounce), which could trigger unintended behaviour without more sophisticated debounce logic.

# 6. Other Modular Programs for Smart Assisted Living

Considering the broader smart assisted living system being built throughout this trimester, the following three modular programs would be valuable additions:

## 6.1 detectMotion()

A motion detection module that reads input from a PIR (Passive Infrared) sensor. This module could automatically trigger lights when motion is detected near doorways or hallways, removing the need for Linda to press a button at all. Keeping this as a separate module means it can be reused across multiple rooms without rewriting the detection logic each time.

## 6.2 monitorTemperature()

A temperature monitoring module that reads data from a temperature sensor and triggers heating or cooling systems when the room temperature falls outside a comfortable range. For elderly residents like Linda, maintaining a safe temperature is critical. Modularising this function means the threshold values can be easily adjusted, and the module can be combined with alert systems independently.

## 6.3 sendEmergencyAlert()

An emergency alert module that detects if Linda has not moved or interacted with the system for an unusually long period of time, and sends a notification to a carer or family member. This module would work independently of the lighting system but could share input data from the motion detection module, demonstrating how modular programs can be combined to create more complex, intelligent behaviours without tightly coupling their logic.